INTERVAL SWITCH # 822 - Parts list (1 of 2) - Revised 02-05-2002 (red)

Mouser unless noted

VR1 to VR11 -	50 ohm 20 turn trim pot			72-T93YA-50

VR12 to VR17 -	100 ohm 20 turn trim pot 		72-T93YA-100

VR18 & VR19 - 	20K ohm 20 turn trim pot		72-T93YA-20K

R1			59 ohm 1%

R2 to R11		33 ohm 1%				1% metal are

R12			2.8K ohm 1% (see note below)		# 271-value

R13			698 ohm 1%

R14			887 ohm 1%

R15			909 ohm 1%

R16 & 17		953 ohm 1%

R18			909 ohm 1%

R19			887 ohm 1%

R20			698  ohm 1%

R21			100 K 1% (hand match to same as R22)

R22			100 K 1% (hand match to same as R21)

R23			180 K 5%

R24               		402 K 1% - matching not required

R25			402 K 1% - matching not required

R26			24 K 1%

R27			100 K 1% (hand match to same as R28)

R28			100 K 1% (hand match to same as R27)

R29			180 K 5%

R30			392 K 1% - matching not required

R31			392 K 1% - matching not required

R32			24 K 1%

R33			10 K 1% (hand match to same as R34)

R34			10 K 1% (hand match to same as R33)

R35			5.1 K 1%

R36			10 K 1% (hand match to same as R37)

R37			10 K 1% (hand match to same as R36)

R38			5.1 K 1%

R39 & R40		100 K 5%

R41			470 ohm 5%

R42			51 K 1%

R43			470 ohm 5%

R44			51 K 1%

Resistors R21, R22, R27, R28, R33, R34, R36 and R37 requiring hand matching
can be supplied with PCBs.

Note: Due to part tolerances, R12 may need to be adjusted to the next size resistor down which
is 2.74K. If I were ordering parts for an 822, I wouldget both the 2.8 K and 2.74 K resistors

INTERVAL SWITCH # 822 - Parts list (2 of 2) - Revised 02-05-2002 (red)

C1 & C2		10 uF electrolytic		140-LLRL-35V10

C3 to C8		0.1 uF ceramic			581-SA-105E-104M

C9 to C12		1n0  yellow box film		581-1N0J63

SW 1 & SW 2      12 position 1 pole rotary sw		10WC204 *

(step switches)    (continuous rotation - no stop)

SW 3 & SW 4         12 position 1 pole rotary sw		105-13571 * *

(octave switches)          (adjustable stop)

* Alternate smaller diameter switch for SW3 and SW4 is:

Electroswitch C6P0112N-A  				Allied # 747-6760

* * Alternate switch for SW1 and SW2 that provides a full ½ inch top clearance

and has no fixed stop so it rotates continuously is:

Electroswitch C6P0112N-C				only from Electroswitch

Contact me if you are looking for the – C version.  I stock them.

U1 to U3		LT1013CP op amp		Newark  06F 9484

or	Allied 735-3671

J1 to J4		Switchcraft 112A			widely available

Misc:

Knobs		Alco # PKES-90B-1/4			Newark 57F2348

Power connectors:	circuit board header:		571-6404454

connector for wire:		571-6404264

white cap cover:		571-6405514

PCB from me - $15.00