INTERVAL SWITCH # 831 - Parts list (page 1 of 2) - Revised 02/09/02 (red)

Mouser unless noted

VR1 to VR10 -	100 ohm 20 turn trim pot		72-T93YA-100

VR11 & VR12 - 	20K ohm 20 turn trim pot		72-T93YA-20K

R1		2.1 K  1%

R2		150 ohm  1% (+5 position = +10 semitones)		1% metal are

R3		51 ohm  1% (+4 position = +7 semitones)		#  271-value

R4		0 ohm (+ 3 pos = +5 semitones)(position for resistor shorted)

R5		0 ohm (+2 pos = +4 semitones)(position for resistor shorted)

R6		200 ohm  1% (+1 position = +3 semitones)

R7		365 ohm  1% (-1 position = -5 semitones)

R8		68 ohm  1% (-2 position = -7 semitones)

R9		0 ohm (-3 pos = -8 semitones)(position for resistor shorted)

R10		232 ohm  1% (-4 position = -12 semitones)

R11		909 ohm  1% (-5 position = -24 semitones)

R12		953  1%

R13		180 K  5%

R14		100 K  1% (hand match to same as R15)

R15		100 K  1% (hand match to same as R14)

R16		499K  1% - no matching required

R17		33 K  1%

R18		10 K  1% (hand match to same as R19)

R19		10 K  1% (hand match to same as R18)

R20		5.1 K  1%

R21		100 K  5%

R22		51 K  1%

R23		470 ohm  5%

R24		180 K  5%

R25		100 K  1% (hand match to same as R26)

R26		100 K  1% (hand match to same as R25)

R27		487 K  1% - no matching required

R28		33 K  1%

R29		10 K  1% (hand match to same as R30)

R30		10 K  1% (hand match to same as R29)

R31		5.1 K  1%

R32		100 K  5%

R33		51K  1%

R34		470 ohm  5%

R35		180 K  5%

R36		100 K  1% (hand match to same as R37)

R37		100 K  1% (hand match to same as R36)

R38		487 K  1% - no matching required

R39		33 K  1%

R40		10 K  1% (hand match to same as R41)

R41		10 K  1% (hand match to same as R40)

R42		5.1 K  1%

R43		100 K  5%

R44		51 K  1%

R45		470 ohm  5%

INTERVAL SWITCH # 831 - Parts list (page 2 of 2) - Revised 02/09/02(red)

C1 to C6		1n0  yellow box film		581-1N0J63

C7 & C8		10 uF electrolytic		140-LLRL-35V10

C9 to C18		0.1 uF ceramic			581-SA-105E-104M

SW 1 to SW 3	12 pos 1 pole rotary switch		105-13571

(adjustage stop)

Alternate switch (higher price) but	Electro-switch		Allied #s

Provides the additional clearance	C6P0112N-A		747-6760
for the top switch that some made	or C5P0112N-A		747-6747

need for wood cabinet type construction

U1 to U5		LT1013CP op amp		Newark  06F 9484

or	Allied 735-3671

J1 to J6		Switchcraft 112A			widely available

Misc:

Knobs		Alco # PKES-90B-1/4			Newark 57F2348

Power connectors:		circuit board header: 	571-6404454

connector for wire: 	571-6404264

white cap cover:	571-6405514

custom PCB from me - $15.00